Course Name: DIGITL LOGIC DES LAB

Course Number and Section: 14:332:233:01

Experiment: Lab1 Report

Lab Instructor: ZAHRA AREF

Date Performed: 9/20/2024

Date Submitted: 9/27/2024

Submitted by: Chance Reyes 225006531, Sion Dutta

Course Name: _________________

Course Number and Section: 14:332:xxx:xx

! Important: Please include this page in your report if the submission is a paper submission. For electronic submission (email or Sakai) please omit this page.

--------------------------For Lab Instructor Use ONLY--------------------------

GRADE: _______________

COMMENTS:

Electrical and Computer Engineering Department

School of Engineering

Rutgers University, Piscataway, NJ 08854

ECE Lab Report Structure

- Purpose / Introduction / Overview – describe the problem and provide background information:

In lab 1, the goal was to implement 2 logic functions, F(A, B, C) and G(A, B, C) on breadboards.

- Approach / Method – the approach took, how problems were solved:

To implement the 2 functions, we used 3 pull up resistors from a 5V rail to be the A B and C inputs. For the logic gates, the 74LS04 (hex inverter/NOT) 74LS08 quad 2-input (AND) 74LS32 quad 2-input (OR) ICs were used. The outputs were then used to turn an LED on or off if the result was 1 or 0 respectively.

- Results – present your data and analysis, experimental results, etc.

Circuit Diagram:

The output of F is labeled on the bottom right. The output of G can be gotten by removing the gates with an X.

Truth Tables:

F(A, B, C) = A. B + A′. C + B. C (7 Logic Gates used):

G(A, B, C) = A. B + A′C (5 Logic Gates used):

According to the truth tables, functions F and G have the same output.

4. Conclusion / Summary – what was done and how it was done

Since F and G have the same output, function F can be simplified using Boolean theorems. By the redundancy theorem, term B.C can be removed and still have the same results as G. This also means that 2 gates can be removed from the circuit diagram of F to get the same output, reducing the total number of gates from 7 to 5. The LED can only turn on if the anode(+) goes HIGH and the cathode(-) goes LOW, so the final NOT gate is optional because it serves to invert the signal for the LED to turn on when the function’s results are HIGH.

.